# LAKEWOOD CULTURAL CENTER

# TECHNICAL SPECIFICATIONS

# Updated: July 2025

# Please disregard all other copies of this document!

# TECHNICAL STAFF

Technical Director			Gabe Paul 	303-987-7883 	gabpau@lakewood.org

Assistant Technical Director		David Kocina 	303-987-7847 	davkoc@lakewood.org

# ADMINISTRATIVE STAFF

Cultural Center Administrator 	Rita Sommers 	303-987-7873 	ritsom@lakewood.org

Facility Coordinator	John Jungerberg	303-987-7976	johjun@lakewood.org

Box Office Manager 	Kim Husted 	303-987-7846 	kimhus@lakewood.org

www.Facebook.com/LakewoodCulturalCenter

# GENERAL INFORMATION

The Lakewood Cultural Center is located on the southwest corner of Wadsworth Boulevard and South Alameda Avenue in the Lakewood City Commons.

# Physical Address of Theater and Box Office:

# GPS Coordinates: 39°42’31” N   105°5’7” W

470 S. Allison Parkway

Lakewood, CO 80226

# Box Office Phone Number:

303-987-7845

Cultural Center Website:

www.Lakewood.org/CulturalCenter

# Directions to the Lakewood Cultural Center:

# From I-25 / 6th Avenue

- I-25 to 6th Avenue

- West on 6th Avenue to Wadsworth Boulevard

- South on Wadsworth to Alameda Avenue

- West on Alameda to S. Allison Parkway

- South on S. Allison Parkway two blocks to the roundabout

# From I-70

- I-70 to Wadsworth Boulevard

- South on Wadsworth to Alameda Avenue

- West on Alameda to S. Allison Parkway

- South on S. Allison Parkway two blocks to the roundabout

# From C-470

- C-470 to Wadsworth Boulevard

- North on Wadsworth to Alameda Avenue

- West on Alameda to S. Allison Parkway

- South on S. Allison Parkway two blocks to the roundabout

# Parking:

Parking is available on the upper deck of the parking garage near the front and stage doors.

Underground parking is accessed from S. Allison Parkway or W. Virginia Avenue.

# Loading/Unloading:

Located off roundabout between rear of building and King Soopers grocery store.

The loading dock is for loading and unloading only. It is not to be used for parking

VENUE INFORMATION

# Theater Type:

# Single balcony, fixed proscenium. Opened 2000, Front of House Renovated 2014. Free wireless internet access is available throughout the venue.

Seating Capacity:

Total Seating:	320	(Includes sixteen accessible seats)

Orchestra Section:	242 	(Includes eight accessible seats)

Balcony Area:	78	(Includes eight accessible seats)

# DRESSING ROOMS

There are two dressing rooms located upstage center in the hallway immediately behind the stage. Both are

equipped with counters, chairs, mirrors, lights, hot and cold water, restrooms, showers, live sound and video feed from stage and a paging system.

# Dressing Room #1:	16-22 Person

Dressing Room #2:	6-12 Person

The Green Room is adjacent to the dressing rooms and is available for whatever may benefit your needs. The room is comfortably furnished and includes a microwave and refrigerator. It is equipped with a paging system and video feed. Free wireless internet access is available.

An iron, ironing board and fabric steamer are located in the Green Room. Six wardrobe racks are available backstage. No laundry facilities are available in the theater.

# STAGE INFORMATION

# Dimensions:

Proscenium Opening:					52’ - 6” Wide		23’ - 4” High

Plaster Line to US Wall:				42’ - 0”

Plaster Line to DS Edge:				2’ - 0”

Stage Width (Wall-to-Wall)				80’ - 0”

Wing Space (SR and SL)				14’ - 0”

Height from Stage to Pipe Grid:			29’ - 0”

Height from Stage to Electric Trim:			27’ - 0"

Height from Stage to Bottom of Catwalks:		25’ - 0"

Height from Stage to Audience Seating: 		3’ - 1”

Crossover Space:

The area between the upstage backdrop and the back-wall MAY NOT be used for crossovers.

Instead, please use the upstage hallway directly behind the stage.

# Orchestra Pit:

The orchestra pit lift is 9’ - 5” deep by 29’ - 8” wide (US) and 37’ wide (DS). Height is fully adjustable.

The orchestra pit is 14’ - 8" deep by 37’ wide, 8’ below stage level and ADA accessible.

# Stage Floor:

Semi-sprung. Maple Tongue and Groove on top of two layers of plywood and neoprene pads placed on 12” centers over four additional layers of plywood. The floor is painted matte black. Nailing or screwing into the stage floor is NOT permitted.

# Loading Dock:

The loading dock is stage level and located upstage left, approximately 10 feet from the stage.

The exterior height of the loading dock is 3’. The exterior loading dock door is 8’ wide x 8’ high.

The interior loading area is separated from the stage by two 14’ wide x 16’ high overhead doors.

There are no dock plates or leveling plates available.

# RIGGING INFORMATION *See rigging criteria at end of packer for load limits.

Two sections of truss are flown upstage and downstage. All other rigging in the theater is dead hung. The pipe

grid is 29’ above the stage floor and consists of a 3’ x 5’ matrix constructed from 1.500” SCH80 steel pipe.

Pipe Grid: Point loads from pipe grid and catwalk handrails may not exceed 100lbs per point. Only one

point may be hung between pipe grid, and handrail support hangers. All rigging is subject to approval.

Structural Steel: Load limit is 2,000lbs per point in very specific locations. Access is from over stage catwalks. For further clarification, or creative solutions, please review rigging criteria page at end of packet and/or contact a member of our technical staff.

# Rigging Schedule (From Plaster Line):

Downstage Truss		-1’ - 0"

Main Valance			0’ - 6”

1st Electric			8’ - 0”

Movie Screen			9’ - 6”

Spare Track (scrim/traveler)  10’

Main Drape			10’ - 6”

Border 1			11’ - 6”

Leg Set 1			13’ - 4”

2nd Electric 			14’ - 7”

Leg Set 2			21’ - 6”

3rd Electric			22’ - 0”

Mid Stage Traveler		24’ - 0”

4th Electric			28’ - 0”

Leg Set 3			28’ - 6”

Black Scrim			35’ - 9”

5th Electric (Empty)		36’ - 6”

Upstage Traveler		37’ - 9”

Cyclorama			38’ - 3”

Upstage Cyc Truss		40’ - 3”

# SOFT GOODS *All soft goods are dead hung from catwalks with exception of star drop

Main Valance		1	Size: 8’h x 74’w	Color: Deep Burgundy	75% Fullness		Main Drape		2	Size: 30’h x 37’w	Color: Deep Burgundy	75% Fullness

Black Legs		6	Size: 30’h x 10’w	Color: Black			50% Fullness

Black Tabs		2	Size: 28’h x 10’w	Color: Black			50% Fullness

Black Tabs		2	Size: 28’h x 4’w	Color: Black			50% Fullness

Travelers		4	Size: 30’h x 34’w	Color: Black			50% Fullness

Borders		6	Size: 6’h x 66’w	Color: Black			50% Fullness

Scrim			1	Size: 30’h x 60’w	Color: Black			Sharks-tooth

Cyc			1	Size: 30’h x 66’w	Color: White			Leno-filled Scrim

Star Drop	1	Size: 14’h/w x 24’h/w	Color: Black *fiber optic

# LIGHTING

# Control Console:	ETC Gio with 2 x 10 Fader Wing.

Control Location:	The Control Booth is located at the rear of the house under the balcony.

House Dimmers:	288 - 2.4K Colortran I-94 dimmers located in dimmer closet on the balcony level.

# Conventional/LED Light Inventory: (20A, Conventional are Stage pin & LED are Edison connectors)

|  | Inventory | In Use | Remaining |
| --- | --- | --- | --- |
| ETC Source Four Body | 120 | 85 | 35 |
| 10° Lens Tube | 2 | 0 | 2 |
| 19° Lens Tube | 18 | 13 | 5 |
| 26° Lens Tube | 54 | 51 | 3 |
| 36° Lens Tube | 60 | 57 | 3 |
| 50° Lens Tube | 10 | 9 | 1 |
| 19° EDLT | 2 | 0 | 2 |
| 26° EDLT | 2 | 0 | 2 |
| ETC Source Four 15/30° ETC Selador Vivid R 63” ETC Color Source DB ADJ Z100 Philips Color Blast 12 | 6 16 39 23 12 | 6 16 39 22 12 | 0 0 0 1 12 |
| Strand Fresnel - 8" | 4 | 0 | 4 |
| Altman Par 64 - WFL | 20 | 0 | 20 |
| Altman 3-Cell Sky Cyc | 4 | 0 | 4 |

NOTE: We keep more lenses in our Source Four inventory than bodies. Please plan accordingly.

# Moving Light Inventory: (20A, L6-20 connectors)

|  | Inventory | In Use | Remaining |
| --- | --- | --- | --- |
| Martin MAC 500 | 2 | 0 | 2 |
| Martin MAC 575 Chauvet Maverick MK2 Martin Mac Aura | 6 7 15 | 6 7 15 | 0 0 0 |

The theater maintains a semi-permanent house hang that may be modified when schedule and labor budget permit. All changes to this light plot must be restored at the production’s expense.

Booms:				Distance from plaster line: (1- Shin, 1- Mid & 1- Head /Boom)

House Hang:		(30)	 ETC Color Source DB		LED

DS Truss:			Distance from plaster line: 1’ – 0"

House Hang:		(5)	ETC Source 4 - 26°		750W

House Hang:		(4) 	Martin MAC 575 Krypton 	575W

1st Electric:			Distance from plaster line: 8’ - 0”

House Hang:		(5)	Mac Aura 11-58° 		LED

House Hang:		(6) 	ETC Source 4 – 36°		575W

House Hang:		(3) 	ETC Source 4 – 			575W

2nd Electric: 			Distance from plaster line: 14’ - 7”

House Hang 		(5) 	Mac Aura- 11-58° 		LED

House Hang		(6)	ETC Source 4 – 36°		575W

House Hang:		(3) 	ETC Source 4 – 			575W

House Hang: 		(2) 	Martin Mac 575 Krypton 		575W

3rd Electric:			Distance from plaster line: 22’ - 0”

House Hang		(5) 	Mac Aura- 11-58° 		LED

House Hang:		(6) 	ETC Source 4 – 36°		575W

House Hang:		(3)	ETC Source 4 – 			575W

House Hang:		(3)	–ETC Color Source DB		LED

4th Electric:			Distance from plaster line: 28’ - 0”

House Hang:		(5) 	Chauvet Maverick MK2 Spot	LED

House Hang:		(5)	ETC Source 4 – 50° 		575W

House Hang:		(6) 	ETC Source 4 – 36°		575W

House Hang:		(3)	ETC Source 4 – 			575W

5th Electric (US Truss):	Distance from plaster line: 40’ - 3”

House Hang:		(8) 	ETC Selador Vivid R 63"	LED

Low Coves:			Distance from plaster line: -3’ - 0”

House Hang:		(8) 	ETC Source 4 – 26°		750W

High Coves:			Distance from plaster line: -3’ - 0”

House Hang:		(8) 	ETC Source 4 – 26°		750W

Box Booms:			Distance from plaster line: -17’ - 0”

House Hang:		(8) 	ETC Color Source DB – 26°	750W

1st Beam:			Distance from plaster line: -15’ - 6” at centerline

House Hang:		(16) 	ETC Source 4 – 26°		750W

Balcony Rail:			Distance from plaster line: -43’ - 6” at centerline

House Hang:		(6) 	ETC Source 4 – 15/30°		750W

House Hang:		(5) 	ETC Source 4 – 19° 		750W

Spot Rails:			Distance from plaster line: -41’ - 6”

House Hang:		(8) 	ETC Source 4 – 19°		750W

Follow Spot Booth:		Distance from plaster line: -47’ - 6”

House Equipment:	(2) 	Lycian Super Arc 400		400W

Ground Row:			Distance from plaster line: -40’ - 3”

House Hang: 		(8)	ETC Selador Vivid R 63" 	LED

*A number of accessories including side arms, pattern holders, drop-in irises, half hats, top hats, barn doors, floor bases and twofers are available. Please contact us for specific inventory.

# THE HOUSE HANG HAS NO STOCK FOCUS OR COLOR

# SOUND

# House Sound System:

Main Speakers		EAW JF-260Z				3 Flown per Side

Subwoofer		McCauley 840 			1 Flown at Center

# Console:

Allen & Heath Avantis	64 input channels fully configurable with 128x128 Dante card

FOH mix position is in the Control Booth located at the rear of the house under the balcony.

No alternative mix position is available.

All audio transport is 96k Dante, there are (2) DT168 stage boxes that can be placed anywhere on stage to accommodate 32 inputs and 16 outputs total. Connections to the Dante network are available in all 4 corners of the stage, both sides of the orchestra pit, spot booth, and on each catwalk.

# Outboard Rack:

# (20) Shure QLX-D wireless microphones

# bodypack transmitter, guitar cable and Shure WL98 tan microphone elements

# (4) Shure SLX Beta58 Wireless Receivers

# (2) Shure ULX Wireless Receivers

# (1) Tascam CD-RW900 CD Recorder

# (2) Furman PL-8 Power Conditioner w/ Lights

# (1) Allen & Heath GX4816 for wireless microphones and CD recorder

# (1) Mac Mini with Q Lab 4 Full Lighting, Video & Audio Package, and Dante Virtual Soundcard

# (5) Hearing Loop Amplifiers

# (1) Cisco SG350 Network Switch (LCC Managed)

# Monitor Speakers:

# (4) EAW SM200 IH-12”

# (4) JBL JRX100

# (4) JBL SF25

# (4) Yamaha SM 12” IV

# (4) Yamaha SM 15” V

# (1) JBL JRX 200

# Amp Rack:

# (1) Allen & Heath AHM64 System Processor with 64x64 Dante Card

# Provides all speaker/matrix processing with remote controls in the Lobby and Control Booth

# (1) Powersoft Audio Quattrocanali 4804 DSP+DANTE (Main Speakers and Sub)

(1) Powersoft Audio Quattrocanali 2404 DSP+DANTE (Monitors)

# (2) Powersoft Audio Quattrocanali 1204 DSP+DANTE (Lobby and Backstage)

# (1) Powersoft Audio Duocanali 4804 DSP+DANTE (Plaza)

(1) Furman ASD-120 Sequenced Power Distro

(1) Cisco Network Switch (City Managed)

# Snakes:

# (2) Whirlwind Medusa 6 Channel

# (2) Pro-Conex Little bro 8 Channel

# (1) Carvin SNK 16XR 16 Channel

Microphones & Direct Boxes:

Wired:

(7) SM58

(4) Audio Technica M4000S

(8) SM57

(2) SHURE- A2WS

(5) Audix D4

(6) RODE M5

(1) BETA 52

(1) ND68

(2) Sennheiser Shotgun Mic’s

(3) PCC Floor Mic’s

(1) PZM Floor Mic

(9) SM81

(3) EU RE 20

(3) MD421

(1) N/D 468

(1) SM87A

(1) Audio Technica PRO4L

(1) Earthworks Piano Mic

(1) MX418 D/C Microflex desktop Mic

Wireless:

# (20) QLX D4 Lavalier Mic’s

# (2) ULX Lavalier Mic’s

# (4) PGX2 SM58 Handheld Mic’s

# (4) SLX2 BETA 58A Handheld Mic’s

Direct Boxes:

(5) DI Model: PDI

(4) Whirlwind

(4) Behringer Ultra DI 100

# COMMUNICATION

# Clear-Com Outlet Locations:

# (3) Control Booth					(2) Follow Spot Booth

(1) Downstage Right					(1) Downstage Left

# (1) House – Row G					(1) Green Room

# (1) Orchestra Pit (Pit Level)				(1) Orchestra Pit (Basement Level)

(10) Catwalks (2 each)

# Hard Wired System:						Wireless System:

(10) Single channel belt-packs				(1) Clear-Com Freespeak II Base Station

(12) Single muff headsets					(5) Clear-Com Freespeak II Wireless Packs

# Closed-Circuit Television:

# Video monitoring is from a pair of Sony BRC-Z330 HD robotic video cameras at the rear of the house which have views from SL & SR SM Panels, Both Dressing Rooms, Green Room, Box Office & two additional Lobby TV’s.

# Assistive Listening System:

Contacta Hearing loop system covering the entire theatre seating area

(2) v14 drivers

(1) v22a Pro Driver

(2) HLD9 Drivers

# PROJECTION

# Front Projection Movie Screen:

24’ W x 20’ H. *Height varies upon request. Movie screen is permanently hung immediately downstage of the main drape and hung 10’ above stage floor for use of downstage area.

System is motorized and may be controlled from stage left or from the control booth.

# Video Projection:

Projectors: EIKI EK-815U (Main Projector) & EIKI LC-X85, 7000 lumens. (Spare)

# Lens: Power zoom and focus, FL: 2.50″ ~ 4.39″ & 2.3-3.3

# A/V Playback:

Blackmagic Video Switcher

27-inch iMac with Retina 5K display, i9 processor, 5.0GHz, 32GB

- Q Lab 4 Full Lighting, Video & Audio Package, and Dante Virtual Soundcard.

Sony Blu-Ray Disc Player.

# VIDEO/LIVE STREAMING

# Cameras:

(3) Panasonic CX-350 with tripods, 7” monitors, Cuebi wireless tally, VariZoom Pro Controller, various lens options & other camera package additions.

Video Switcher:

(1) Blackmagic Television Studio HD

Video Encoder:

(1) Taradek Video Cube 655

# POWER

Service (May be used at production’s expense. Bare-end tails required. Disconnects located on MSL wall):

(1) 200A, 208Y/120 VAC 3Ø Disconnect.

NOTE: The above 200A disconnect is used to supply power to the house moving light circuits.

- 400A, 208Y/120 VAC 3Ø Disconnect.

# MISCELLANEOUS EQUIPMENT

# Manlift:

Genie AWP-25S	Platform Height: 25’ - 0”	Working Height: 30’ - 0”	Lift Capacity: 350 lbs

# Ladders:

16’ Werner Fiberglass Extension Trestle Ladder

12’ Werner Fiberglass Step Ladder

8’ Werner Fiberglass Step Ladder

4’ Werner Aluminum Step Ladder

2’ Werner Aluminum Step Ladder

# *Marley Floor:

Harlequin		Black/White Reversible	6 panels @ 78.5” wide x 50’ long

Rosco			Black/Gray Reversible	7 panels @ 63” wide x 52’ long

# *Platforms:

4			4’ x 8’ Wenger Versalite	16” and 8” Legs

# *Choral Risers:

3			Wenger Tourmaster		3-Step, 6’ Length

# *Acoustical Shell:

7			Wenger Travelmaster		9’ - 4” – 12’ - 2” Adjustable Height

# Musician Equipment:

54			Wenger Musician Chairs, Non-Padded

45			Wenger Roughneck Music Stands

1			Wenger Conductor’s Music Stand

1			Wenger Conductor’s Podium

1			Wenger Conductor’s Chair

# *Backline Equipment:

1			Yamaha GH2 5’ - 8" Classic Baby Grand Piano

1			Gallien-Krueger NEO 410 Bass Cabinet

1			Gallien-Krueger 700RB-II Bass Head

1			Strony STR-4 Digital Theater Organ

# Fog/Haze:

1			DF-50 Diffusion Hazer

2			Antari Ice Low Lying- Fog Machine *Dry ice required for use.

- Additional rental rates apply.

.